Scope of Work:

Union Station – Washington D.C. - Display Replacement

PROJECT NAME: Union Station – Display Replacement

ADDRESS:  Union Station, 50 Massachusetts Ave NE, Washington, DC 20002

DATE: Dec 11, 2025									Revision #1

PROJECT OVERVIEW

The following Scope of Work is intended to be general in nature. The intention is to have the successful Subcontractor perform all related work shown on the Contract Documents other than those items specifically indicated below to be excluded.

This Scope of Work takes precedence over the Drawings and Specifications in the event of a conflict in trade assignment or responsibility. By accepting this Scope of Work, the Subcontractor is verifying that the Drawings and Specifications clearly identify the Subcontractor's work.

Objective

Demolition and Installation of the Spectacular displays, Escalator displays and Digital Dios displays at Union Station in Washington, D.C.

Installation

Installation is to take place between 04/01/26 and 06/31/26. Installation of the LED video board included in the equipment list below and described within the SOW is to be part of this scope. All work to be performed during off hours / night work.  All work to be union labor.

Electrical Connection

Electrical connections between LED panels (jumps) will be included as part of this scope of work. “Jumps” refers to both high voltage power and low voltage data connections between ALL LED panels.

Testing and Adjusting

Sub-contractor will adjust the displays to ensure the best quality installation possible and will work with ANC to complete and make any necessary changes.

ANC Signoff

Each display will be reviewed to confirm that equipment has been installed per the SOW and to the satisfaction of the ANC project manager and the customer.

SCOPE OF WORK

- General Inclusions:

  - Digital Dios Displays (Qty 6) including the following elements:

    - Demolition and disposal of existing display.

    - Fabrication and installation of secondary structure consisting of aluminum channel for LED mounting.

    - LED cabinets (Approx 30 cabinets) (5 Rows of 6 Cabinets) per display (8.20’h X 9.84’W)

  - Escalator Displays (Qty 2) including the following elements:

    - Demolition and disposal of existing display.

    - Fabrication and installation of secondary structure consisting of aluminum channel for LED mounting.

    - LED cabinets (Approx 14 cabinets) (2 Rows of 7 Cabinets) per display (3.28’h X 11.48’W)

  - Spectacular Displays (Qty 5) including the following elements:

    - Demolition and disposal of existing display.

    - Fabrication and installation of secondary structure consisting of aluminum channel for LED mounting.

    - LED cabinets Display 1 (Approx 50 cabinets) (5 Rows of 10 Cabinets) per display (8.20’h X 16.40’W)

    - LED cabinets Display 2 (Approx 45 cabinets) (5 Rows of 9 Cabinets) per display (8.20’h X 14.76’W)

    - LED cabinets Display 3 (Approx 70 cabinets) (5 Rows of 14 Cabinets) per display (8.20’h X 22.96’W)

    - LED cabinets Display 4 (Approx 28 cabinets) (4 Rows of 7 Cabinets) per display (6.56’h X 11.48’W)

    - LED cabinets Display 5 (Approx 110 cabinets) (5 Rows of 22 Cabinets) per display (8.20’h X 36.08’W)

  - Power and low voltage cables for display.

- General Exclusions:

  - Temporary power

  - Site security

  - Structural and Electrical engineering

  - Building permits

  - Sidewalk / Lane Closures

  - LED Disposal

- Displays – QTY 1:

  - Removal and disposal of existing displays, and secondary steel (as needed)

  - Unload, receive, inspect and stage all structural components in pre-arranged staging locations (locations will be coordinated by ANC and Owner).

  - Provide manpower and equipment for structural installation displays and any necessary sub structure.

  - Provide floor protection as required by the project / general contractor / engineers.

  - Unload, receive, inspect, and stage LED video panels.

  - Uncrate and install LED video panels.

  - Provide manpower and equipment for structural installation of trim and flashing.

  - Breakdown and dispose of crates as they are unloaded. Disposal is part of this scope of work.

  - Complete all electrical power and low voltage data jumps between assembled LED.

ITEMIZED PRICING

The itemized pricing for the above Scope of Work should be provided in the format below as requested in the RFP documents. If you have any questions, please do hesitate to send over.

BID DUE DATE: Please submit your bid via email to the ANC Contacts ASAP.